EXPERIMENT -16

SIMULATION AND ANALYSIS OF UNIFORM AND NON-UNIFORM QUANTIZATION USING MATLAB

Code:

clc;

clear;

close all;

% Input Signal

t = 0:0.001:1;

x = sin(2*pi*5*t);

% Uniform Quantization (8 Levels)

L = 8;

x_uni = round((x+1)*(L-1)/2);

x_uni = (x_uni*2/(L-1))-1;

% Non-Uniform Quantization (μ-law)

mu = 255;

x_comp = sign(x).*log(1+mu*abs(x))/log(1+mu);

x_non = round((x_comp+1)*(L-1)/2);

x_non = (x_non*2/(L-1))-1;

% Plot Results

figure;

subplot(3,1,1)

plot(t,x)

title('Original Signal')

grid on

subplot(3,1,2)

stairs(t,x_uni)

title('Uniform Quantization')

grid on

subplot(3,1,3)

stairs(t,x_non)

title('Non-Uniform Quantization (\mu-law)')

grid on

Output :